JIP4 PCR 

1. Mastermix: total reaction volume 25 (l

	Stock solutions
	1 reaction ((l)
	X reactions

	DNA
	2
	

	10x Tsg buffer
	2.5
	

	25 mM MgCl2
	2.5
	

	20 (M CS187 
	0.8
	

	20 (M CS188
	0.4
	

	20 (M CS189
	0.4
	

	DMSO
	2.5
	

	10 mM dNTP
	1
	

	H2O
	12.65
	

	Tsg polymerase
	0.25
	


2. Use eppendorf PCR JIP4

95 (C 
3 min

1 cycle
94 (C 
40 sec

62 (C 
45 sec

35 cycles
68 (C
1 min 

72 (C 
10 min

1 cycle

4 (C 
hold

3. Run 2 % agarose gel

425 bp band 

=  -/- (JIP4 KO)

675 + 425 bp bands
= +/- (HET)

675 bp band

= +/+ (WT)

4. Primer sequences are

CS187J4F CTTTGCTACCACAAGTCTTTCATCC
CS188PUROR CTCTCGTGGGATCATTGTTTTTCTC

CS189J4WT TCCTCGCCACTAAGAGTCTTGG 

