DNA TAIL MACHINE

1. DOUBLE HIGHLIGHT QUIASOFT M-48 VER.2.0 ON DESKTOP

2. HIGHLIGHT ON START

3. TISSUE SELECT-HIGHLIGHT ON SELECT 

4. SELECT ELUTION TYPE-1.5ML

5. SELECT NUMBER OF SAMPLES-MUTIPLES OF 6

6. SELECT NEXT

7. CONFIRM TIP GUARD AND WASTE CHUTE CLEAN-HIGHLIGHT NEXT

8. DO YOU WANT TO RESET TIP RACKS?-“YES” IF YOU’VE ADDED MORE TIPS TO THE RACK OR “NO” IF YOU LEFT THE RACKS THE SAME-HIGHLIGHT NEXT

9. SHOWS AMOUNT OF SOLUTIONS AN DWHERE LOCATED ON RACK –MAKE SURE THAT THERE ISN’T ANY PRECIPTATE IN MTL OR MW1

10. MAKE SURE GUARD IS DOWN ON SOUTIONS, MAKE SURE ALL TUBES IN THE RACKS ARE DOWN

11. HIGHLIGHT NEXT

12. MEMO-HIGHLIGHT NEXT

13. SAMPLE PREP-HIGHLIGHT NEXT

14. TUBES-HIGHLIGHT NEXT

15. TUBES (LABELED)-HIGHLIGHT NEXT

16. DO YOU WANT TO ENTER NAMES-HIGHLIGHT “NO”

17. HIGHLIGHT CONFIRM

18. CLOSE THE DOOR-HIGHLIGHT “OK”

19. HIGHLIGHT “GO”

     AFTER RUN

1. HIGHLIGHT NEXT

2. UNDER MANUAL, HIGHLIGHT UNLOCK DOOR

3. PROMPT-“DO YOU WANT TO STERILIZE?”-“YES” IF DONE FOR THE DAY, “NO” IF NOT

4. PROMPT-“DO YOU WANT TO SHUT DOWN G-48-“YES” IF DONE FOR THE DAY, “NO” IF NOT

5. REMOVE EVERYTHING FROM THE MACHINE (EXCEPT THE TIPS)

6. PUT COVERS ON CONTAINERS OR RINSE CONTAINERS OUT WITH DISTILLED WATER

